Fitchburg Massachusetts PM6
2.6 meter, 1500 feet per minute, 150+ TPD, 70 gsm to 180 gsm filled printing papers (42 gsm to 135 gsm fiber)  
Substantially new in 1995.  Additional rebuilds since.
Headbox:
· Voith K airpad box with two rectifier rolls.  Automatic CD basis weight control.  New in 1992.  Stainless Steel.  113” Pond (2.8 m)
· Voith 200 Centriscreen, installed in 2005.

· Celleco cleaning system

Forming Table:

· Stainless steel cantilevered Bellmer fourdrinier new in 1994.

· Breast roll shake.

· Gravity drainage section followed by low vacuum boxes.

· Driven Dandy new in 1994, manufactured by Finch.

· New quick change flat boxes made by Appleton.  New in 2002.

· Double chamber stainless steel couch vacuum roll with new double doctor.

Press Section:
· Dual press manufactured by Rice Barton.
· 1st press is suction with bottom felt.

· 2nd press is solid roll with bottom felt.

· Hydraulic controls replaced in 2000.

Dryers: 

· Major rebuild in 1995.
· Steam/condensate system by Valmet with thermocompressor control.

· High efficiency hood, pocket ventilation, and economizer by Valmet

· All cans are coded
· Main section dryers new in 1995, by Valmet

· Split into small groups for improved drying control.

· 25 Dryer cans, 4’ diameter, 114” width, 75 psi

· New guides, automatic stretches by Bellmer in 1995.

·  After size section
· 9 Dryer cans, 4’ diameter, 114” width, 100 psi.

Size Press:
· Patton Manufacturing

Calender:
· Hunt & Moscrop (Voith) soft nip calendar, new in 1992.

· 2 calendars with 2 rolls each.
· Oil heated hard rolls with Tungsten Carbide coating.

· Sheet cooling cans.

· Sheet moisturizer.

Reel:

· Rice Barton horizontal reel.

· Camera based sheet defect detector system.

· On line process control system by Honeywell.
Winder:

· Webco dual drum drive shaftless winder with regenerative braking system.  New in 2002.  4500 fpm.

Machine Drive:

· Machine drive is DC on all sections, provided by Rockwell Automation in 2000.  Allen Bradley PLC controls.
