Tourmaline – Erik Lyneborg

Location: Northland Debolt Storage Yard

2 x 50 MMSCFD Amine Plants (100 MMSCFD combined)

2019 Startec Unit 18061 & 18062

One (1) Process Skid A (22’ W x 60’ L Skid):

One - Inlet Filter Coalescer (Optional Deduct);

One - Rich Amine Flash Drum;

One - 100% Rich Amine Particulate Filter;

One - 125% Lean/Rich Amine Exchangers;

Two - 100% Lean Amine Booster Pumps;

Two - 100% Lean Amine Charge Pumps;

One - 20% Slip Lean Amine Particulate Filter;

One - 20% Slip Lean Amine Charcoal Filter;

One - Anti-Foam Chemical Injection Pump;

One - Amine Reflux Accumulator;

Two - 100% Amine Reflux Pumps;

Two - 35 kW Building Heaters.

Shipped Loose/Off-skid Items:

One - 4 MMBTU/Hr Fired Amine Reboiler

One - Amine Contactor Tower c/w down-comers;

One - Lean Amine Cooler;

One - Amine Reflux Condenser.

H2S outlet content of <4ppm

55 USGPM flow rate of Huntsman MS100 (to maximize CO2 Slip) or 45wt% generic MDEA.

INLET CONDITIONS

Inlet Gas Flow Rate 50 MMSCFD

Amine Inlet Gas Pressure 1280 - 940 psig

Inlet Gas Temperature 120°F

GAS ANALYSIS AT AMINE PLANT INLET

Mole Fraction [%]

WATER 0.00

HELIUM-3 0.01

NITROGEN 0.40

CARBON DIOXIDE 1.50

HYDROGEN SULFIDE 0.0748

METHANE 82.69

ETHANE 7.64

PROPANE 4.25

ISOBUTANE 0.53

n-BUTANE 1.49

ISOPENTANE 0.35

n-PENTANE 0.47

n-HEXANE 0.32

n-HEPTANE 0.29