# Technical Data – ARBURG ALLROUNDER 420 C

Machine Type: 420 C 1000-250

Clamping Unit:

Clamping force: 1000 kN

Opening force: 35/250 kN

Opening stroke: 500 mm

Mould installation height (min): 250 mm

Distance between platens (max): 750 mm

Tie bar spacing: 420 x 420 mm

Clamping plates (w x h): 570 x 570 mm

Max mould weight (moving half): 600 kg

Ejector force: 40 kN

Ejector stroke: 175 mm

Hydraulics / Drive:

Hydraulic pump power: 18.5 kW

Installed total power: 27.1 kW

Electrical Cabinet:

Safety standard: DIN EN 60204

Socket (Schuko): 1 x 10 A

Socket (Cekon): 1 x 16 A

Injection Unit (250):

Screw diameter: 30 / 35 / 40 mm

Effective screw length (L/D): 23.3 / 20 / 17.5

Screw stroke: 150 mm

Calculated shot volume: 106 / 144 / 188 cm³

Shot weight (PS): 97 / 132 / 172 g

Injection pressure: 2470 / 1820 / 1390 bar

Injection flow rate: 144 / 196 / 256 cm³/s

Injection flow with accumulator: 350 / 476 / 622 cm³/s

Back pressure: 350 / 140 bar

Screw speed: 49 / 58 / 66 rpm

Screw torque: 420 / 490 / 560 Nm

Nozzle contact force: 70 kN

Nozzle stroke: 240 mm

Heating:

Cylinder heating: 1600 + 3x1300 W

Nozzle heating: 600 W

Other:

Material hopper capacity: 50 L

Max horizontal injection position: 170 mm

Dimensions & Weight:

Oil filling: 230 L

Net weight: 3180 kg

Electrical connection: 100 A