OVER ALL CAMERA  :

| Eca HYTEC Explosion proof panoramic viewing color TV camera DTR 100 Z ADF | 10x zoom and tilt / rotate viewing head for panoramic viewing Certified EEx d llC T6 according to ATEX 94/9/EC directive. Other certifications : see data sheet compact (ø 120 mm - L 265 mm) |  |
| --- | --- | --- |
| Eca HYTEC TV Explosion Proof Multiplexed COLOR CCTV SYSTEM MODEL DTR100 ZADFM | W/ SWIVEL HEAD AND ZOOM CAMERA ¦¦2G/D Ex d¦¦C-40/+100`c PT B00ATEX 1059 ISO M20 |  |
| Eca HYTEC Smart color TV camera VS 3673 | -PAL or NTSC video signal - OD 75 mm ; L 145 mm - Controlled by RS 232 serial link or by analogic mode - Operating depth : 1 000 meters - Stainless steel housing |  |
| HY 9806900 |  |  |
| Eca HYTEC Smart color TV camera with 10x zoom Underwater halogen light VS 3673 | -PAL or NTSC video signal - OD 75 mm ; L 145 mm - Controlled by RS 232 serial link or by analogic mode - Operating depth : 1 000 meters - Stainless steel housing Also available in 3 000 meters version |  |
| HYTEC Camera VS 662 ADF | VS 662 ADF EEx d ¦¦C T4 INERIS 93.C.5062 X 0` de fabrication 004 P max 8.5 Watts Umax 14 VAC NE PAS00VRIR SOUS TENSION |  |
| HERNIS Scan System optical magnification superseding typical compact cameras with 36X zoom CCTV Camera Ex 285 PT | ARENDAL NORWAY NO – 229 EEX de ¦¦C T6 IP 66 Certificate no Ex 960311 |  |
| HERNIS Scan system Ex proof Camera Stations HERNIS dedicated CCTV control panel, video wall, OEM HMI (e.g.drillers chair), Ex monitors, touch monitors and mobile units. 8. Control Ex 286 | S No .1256 CE 0575 EX ¦¦2 6 EEx de ¦¦C T6 IP 66 Umax : 40 VAC 50/60 Hz iMAX : 4A Prod Year - 2006 Certificate no - DNV – 2003-OSL-ATEX-0366 IEC Certificate no – IEC EX DNV 06.0002 Ex DE ¦¦C T6 IP 66 |  |
| HERNIS offers a well proven range of CraneTV systems meeting the requirements of Crane operators worldwide the joystick or push button controls. An oil damper keeps the camera housing in a stable vertical position Ex 291C | S no - 30559 CE 0575 Ex ¦¦ 2G Ex d ¦¦C T6 IP 66 Tamd = 50`c +60C Pred Year - 2012 Certificate no - DNV 2003-OSL AT EX -0013X IEC Certificate no - IEC Ex DNV – 06 .0003x |  |
| HERNIS Modular design Internal cabling Integrated junction box Flexible mounting Electro polished stainless steel 316L Robust gearbox Electro-mechanical brakes on shaft PT36W | Material: Stainless steel Surface: Electro polished Pan speed: 0-40/sec Tilt speed; 0-20/sec Equipment: Receiver, Washer/ Wiper 1/3" NTSC / PAL camera, High Resolution, 10 x or 15 x zoom lens, can be controlled from HE 250/ HE 400 Approval: IP66 |  |
| ELSHAM WOLD SRIGG FEH-B-1 | EEx d ¦¦ B T6 BASEEFA CERT NO Ex 88B 1253 VOLRS 230 AC 50Hz MAX POWAR - DISSPATI ON. 11.5 S. No. - 2558 |  |
| Electro-flow Controls LTD EFC GROUP TYPE 556 CAMERA HOUSING CAN BE SUPPLIED AS EITHER A STAND ALONE HOUSING OR A COMPLETE ASSEMBLY WITH CAMERA, LENS, SUNSHIELD AND GLANDED CABLE TAIL. CAMERA UNIT 556 | S. NO. - 211032 Overall Weight; 12kg. Ingress Protection - Rating; IP66. Temperature Rating -; -20°C to 60°C. Heater / Thermostat -; 110/240VAC, 50/60Hz 15 Watts. ITS .10ATEx 1696X / IECEx Its 10.0029X |  |
| Electro-flow Controls LTD EFC GROUP TYPE 556 CAMERA HOUSING CAN BE SUPPLIED AS EITHER A STAND ALONE HOUSING OR A COMPLETE ASSEMBLY WITH CAMERA, LENS, SUNSHIELD AND GLANDED CABLE TAIL CAMERA UNIT 556 | S. NO. - 29016 Overall Weight; 12kg. Ingress Protection - Rating; IP66. Temperature Rating -; -20°C to 50°C. Heater / Thermostat -; 110/220VAC, 50/60Hz 15 Watts. Demko – 03A Tex 135182 |  |
| Electro-flow Controls LTD THE 558 PAN & TILT IS AVAILABLE IN THREE MODES; PAN ONLY, TILT ONLY AND PAN & TILT. THE ASSEMBLY IS FOR INTEGRATION WITH EFC GROUP 556 CAMERA HOUSING AND THE 557 WIPER UNIT. 558 | S. NO. - 21026 Overall Weight; 30kg. Operating Temperature; Ambient -20 to +60°C For Colder Operating Temperatures in the Range of -40°C to -20°C,. Standard Power Requirements; 110v/240VAC, 50-60Hz. Rated Consumption; 50 Watts. Ingress Protection; IP66. Certification; ATEX & IECEx. ITS .10ATEx 1696X / IECEx Its 10.0029X |  |
| Electro-flow Controls LTD THE 558 PAN & TILT IS AVAILABLE IN THREE MODES; PAN ONLY, TILT ONLY AND PAN & TILT. THE ASSEMBLY IS FOR INTEGRATION WITH EFC GROUP 556 CAMERA HOUSING AND THE 557 WIPER UNIT. 558 | S. NO. - 211028 Overall Weight; 30kg. Operating Temperature; Ambient -20 to +60°C For Colder Operating Temperatures in the Range of -40°C to -20°C,. Standard Power Requirements; 110v/240VAC, 50-60Hz. Rated Consumption; 50 Watts. Ingress Protection; IP66. Certification; ATEX & IECEx. ITS .10ATEx 1696X / IECEx Its 10.0029X |  |
| Electro-flow Controls LTD THE 558 PAN & TILT IS AVAILABLE IN THREE MODES; PAN ONLY, TILT ONLY AND PAN & TILT. THE ASSEMBLY IS FOR INTEGRATION WITH EFC GROUP 556 CAMERA HOUSING AND THE 557 WIPER UNIT. 558 | S. NO. - 210034 Overall Weight; 30kg. Operating Temperature; Ambient -20 to +60°C For Colder Operating Temperatures in the Range of -40°C to -20°C,. Standard Power Requirements; 110v/240VAC, 50-60Hz. Rated Consumption; 50 Watts. Ingress Protection; IP66. Certification; ATEX & IECEx. ITS .10ATEx 1696X / IECEx Its 10.0029X |  |
| The Kongsberg Maritime model oe1324 Enhanced SIT camera combines the proven underwater pressure housing with 3,000 metre depth rating as standard (other housing options available). The standard camera includes adaptive contrast enhancement, automatic tube protection circuits, over-voltage protection, and special high-frequency most successful range of professional underwater cameras. OE 1367MKll | Water Depth 3,000 metres, deeper options available Temperature Operating -5°C to +40°C Storage -20°C to +60°C Vibration 10g, 20 - 150Hz, 3-axes (non-operating Size Diameter 102mm exc. guard Length 297mm exc. connector Weight Air 4.6 Kg Water 2.2 Kg Standard Housing 6Al/4V ASTM B 348 Titanium Alloy Signal to Noise >40dB weighted (AGC off) Scanning 625 Line / 50 Hz CCIR 525 Line / 60 Hz RS-170 (optional) Power Input Constant Voltage 16V - 24V dc, Model OE 100 |  |
| The Kongsberg Maritime 14-366-0694Colour Zoom Camera is our industry standard ROV inspection camera. This camera uses the latest super HAD CCD technology to give improved light sensitivity and image definition. The standard depth rating is 4,500 msw. 14-366-0694 | Dimensions Diameter: 88mm Length: 210mm (excl. connector) Weight In air 2.6Kg, in water 1.44Kg Housing Material Titanium Alloy, 6AL/4V ASTM B3 48 Operating Depth 4,500 msw (other depth rated housing options available) Temperature Operating: -5 to 40°C, Storage: -20 to 60°C Shock 30G peak acceleration, 25ms half sine duration, on all three axes Vibration 10G, from 20 to150Hz on all three axes |  |
| The Kongsberg Maritime Seahawk Integrated colour Television System Surface control unit OE 1260A | Temperature Operating 0°C to +40°C Storage -20°C to +60°C Vibration 10g, 20-150Hz, 3-axes (non-operating) Shock 30g peak, 25mS half-sine pulse Mechanical Width 442mm (17.4”) Height 443mm (17.4”) Depth 480mm (18.9”), 392mm (15.4”) Weight 30.0Kg |  |
| Technor as TNCN-121009 | Certification; ATEX & IECEx. Demko - 86-80151. Nemko - 83036. S/N - 10976 |  |
| ROES 06 081606Ex | P/N – 495069 Max.: 4 mm2 Nom.voltage – 120V EEx e ll T6 Ex |  |
| Remote Ocean System Spectator TI NTSC Bipolar 52-26900-A01 | 2 Pcs |  |
| Remote Ocean System Low Light B+W CAM 1500 m ETA 52-23813-01A |  |  |
| Remote Ocean System Navigator TI 3 KM ETA LOC LIA 52-24922-AT06 |  |  |