General Information:

| Customer: Tri-Logic Service | Date: 06/09/20 |
| --- | --- |
| Address: 555 Cannelton Rd., Darlington, PA | PO #: N/A |
| Work Order #: N/A | Unit Description: Slug Catcher |

Examination Information:

| Examination Technique: UT-100 | Material Type: CS | Surface Condition: As Prepared |
| --- | --- | --- |
| NDT Procedure: SOP-122001 | Scanning: See Examination Data Table | Scanning Extent: 100% Areas Identified |
| Technique: Manual | Reference dB: 31.6 dB | Scanning dB: Plus 20 |
| Screen Range: 2”+ | Offset: .237 µsec | Velocity: .2312 in/µsec |

Equipment Used:

| ID Number / S/N | Description | Standard Calibration Date | Standard Calibration Due Date |
| --- | --- | --- | --- |
| SCNDT-2 | GE USM GO+ | 06/09/20 | 06/09/20 |
| 037908 | 1018 Steel Calibration Block | N/A | N/A |

overall

inlet

| General Location | Specific Location | Rotational Location | Nominal Diameter (in.) | Lowest Measurement (in.) |
| --- | --- | --- | --- | --- |
| Inlet | A | 0 deg | 18 | 2.801 |
| Inlet | A | 90 deg | 18 | 2.802 |
| Inlet | A | 180 deg | 18 | 2.803 |
| Inlet | A | 270 deg | 18 | 2.797 |

PSV

| General Location | Specific Location | Rotational Location | Nominal Diameter (in.) | Lowest Measurement (in.) |
| --- | --- | --- | --- | --- |

| PSV | B | 0 deg | 6 | 1.649 |
| --- | --- | --- | --- | --- |
| PSV | B | 90 deg | 6 | 1.636 |
| PSV | B | 180 deg | 6 | 1.660 |
| PSV | B | 270 deg | 6 | 1.646 |

Manway

| General Location | Specific Location | Rotational Location | Nominal Diameter (in.) | Lowest Measurement (in.) |
| --- | --- | --- | --- | --- |

| Manway | C | 0 deg | 26 | 3.864 |
| --- | --- | --- | --- | --- |
| Manway | C | 90 deg | 26 | 3.879 |
| Manway | C | 180 deg | 26 | 3.880 |
| Manway | C | 270 deg | 26 | 3.871 |

outlet

| General Location | Specific Location | Rotational Location | Nominal Diameter (in.) | Lowest Measurement (in.) |
| --- | --- | --- | --- | --- |

| Outlet | D | 0 deg | 18 | 2.794 |
| --- | --- | --- | --- | --- |
| Outlet | D | 90 deg | 18 | 2.787 |
| Outlet | D | 180 deg | 18 | 2.721 |
| Outlet | D | 270 deg | 18 | 2.796 |

Under no circumstances shall Steel City NDT, LLC be liable for any incidental, special, punitive, or consequential damages arising out of or in connection with this agreement or the services performed hereunder, including but not limited to, liability for lost profits, business interruption, or loss of business.  The maximum total liability of Steel

City NDT, LLC arising out of or relating to the services performed under this agreement, regardless of the form of action, whether in contract, statute, tort (including, without limitation, negligence), or otherwise, shall in no event exceed the fees paid to Steel City NDT, LLC hereunder. Inspection results only valid at time of inspection. Areas inspected were selected by Tri-Logic Service and in no way gives validation of the thickness outside of the inspection areas.

| Form Sign-Off | Signature | Level | Date |
| --- | --- | --- | --- |
| Technician | Eric Mateya | II | 06/09/20 |

Inspection Pictures: