SIMAC “RED RING” HOUSING LESS

TYPE: HORIZONTAL AND VERTICAL STANDS

THE EXISTING MATERIALS OFFERED ARE COMPLETELY REVAMPED

The Available parts of a roughing section that can use billets up to 160 mm X 160 mm size is composed of:

N.1 HORIZONTAL VERSION

Cylinder stands:

- Cylinder diameter 600 mm – barrel diam. 800 mm – neck diam. 280 mm

- Cylinder grooved for billet 120-130-140-150-160 mm

- Housing Cast Steel complete with Hydraulic fixing bracket

- Lubrication system for Bearing

- Set of entry and exit guide

- Cooled Water pipe system for cylinder

Gear box – pinion stand units (GPR):

- Distance axe to axe 530 mm. properly ratio

- Body in steel welded and gear teeth high resistance hardened cemented,

- Complete with gear couplings in entry and coupling box in exit.

- Service factor 2

Cardanic spindles

Motor: DC MOTOR ‘ Electro Mecanique / Brown Boveri ’

- Power 445 Kw. 750 Volt complete with electrical panels and accessories.

N.1 CONVERTIBLE TYPE H/V

Cylinder stands:

- Cylinder diameter 600 mm – barrel diam. 800 mm – neck diam. 280 mm

- Cylinder grooved for billet 120-130-140-150-160 mm

- Housing Cast Steel complete with Hydraulic fixing bracket

- Set of entry and exit guide

- Lubrication system for Bearing

- Cooled Water pipe system for cylinder

Gear box – pinion stand units (GPR):

- Distance axe to axe 530 mm. properly ratio

- Body in steel welded and gear teeth high resistance hardened cemented,

- Complete with gear couplings in entry and coupling box in exit.

- Service factor 2

Cardanic spindles

N. 3 HORIZONTAL/VERTICAL VERSION

Cylinder stands:

- N.5 Cylinder diameter 600 mm – barrel diam. 800 mm – neck diam. 280 mm

- Cylinder grooved for billet 160 - 170

- Housing Cast Steel complete with Hydraulic fixing bracket

- Set of entry and exit guide

- Lubrication system for Bearing

- Cooled Water pipe system for cylinder

Cardanic spindles

Lubrication unit

Roll change robot for mill stands

The following documentation will be included in the supply:

- Hydraulic diagrams with list of components

- Electric circuits with components list

- Functional works

- Bearing list

- Spare parts list

- Motor list

- Sensor list

ENGINEERING

- Includes Lay-out drawings for machine positioning

- Load foundation drawings on them

- Machine assemblies

TERMS & CONDITIONS

DELIVERY TIME

6 months

DELIVERY TERMS

Ex work

PACKAGING

Included in sea containers od 20 or 40 feet

TERMS OF PAYMENT

50 % Advance Payment

50% Against shipping documents

The following offer regards what is available at the moment; anyway is possible to change or modify some part for being suitable to the customer’s needs.