SPECS OF UNIT

Manufacturer: Production Facilities Equipment Co., Inc.
Serial #: VS-3018-0118

This unit is a:

## Truck-Mounted LACT Separator (Skid Unit)

From auction/field data, that exact serial number corresponds to:

- 18" OD vessel

- Approx 4 ft x 15 ft separator body

- Skid-mounted package

- Designed for LACT (Lease Automatic Custody Transfer) service

### Separator Vessel

- Oil/water separation before measurement

- Non-code vessel (common for field units)

### Pump System

- Gorman-Rupp gear pump

- ~3 HP motor (Class I Div 2 rated)

- 2.5" NPT connections

- Built-in relief (30–95 PSI range)

### Skid

- Open oilfield skid

- Roughly 7.5' x 6.5' footprint

What It’s Used For

This is standard in:

- LACT units (custody transfer)

- Tank batteries

- Truck loading/unloading

- Oilfield production sites

Basically:
Separates fluid → pumps it → sends it through meter for sales

Rosemount 3051CD Differential Pressure Transmitter

3051CD = Coplanar Differential Pressure Transmitter

Accuracy: up to ±0.075% of span

Output: 4–20 mA + HART

Pressure capability: varies by range (often up to 2,000+ psi line pressure)

Long-term stability: designed for 15+ years service

### Tornado Meter (Calgary, Alberta, Canada)

- Positive displacement / inline flow meter or flow element

- Used in LACT measurement systems

Measures fluid volume very accurately before custody transfer

## Pump Package

- Gorman-Rupp gear pump

- Electric motor driven ( ~3 HP)

- Moves product through the meter