| VF-2 | Vertical Machining Center; 30" x 16" x 20" (762 x 406 x 508 mm), 40 taper, 30 hp (22.4 kW) vector drive, 8100 rpm, inline direct-drive, 20-station carousel tool changer, 1000 ipm (25.4 m/min) rapids, 1 GB program memory, Wi-Fi Connection for the Haas Control, 15" color LCD monitor, Parameter Unlock Option, USB port, ethernet interface, early power failure detection module, memory lock keyswitch, rigid tapping and 55-gallon (208 liter) flood coolant system. |
| --- | --- |
| TRANS/45KVA | 3 Phase / 60 Hz, 45 KVA, 600/230 Isolation Transformer |
| 10K-40T-GREASE-IN | 10,000-rpm Grease-Packed Spindle, 40 taper, 30 hp (22.4 kW) vector drive, inline direct-drive; grease lubrication in place of the standard air/oil mist. |
| AUGER | Chip Auger |
| AAG | Automatic Air Gun |
| AUTO DOOR-MILL | Auto Door |
| EDGE SENSOR | Edge Sensor: stops moving parts upon contact to prevent injury. For use with auto doors and other automated systems. NOT compatible with ROBOT or APL. |
| HAAS CHIP LIFT | Haas Chip Lift; conveys chips from the auger chute on the machine into a 55-gallon drum or hopper. Ideal for high-volume chip production or machine cleanup. |
| PROG AIR-DUAL VISE | Programmable Air for Dual Vises; allows users to operate up to two pneumatic devices on their machine via the Haas control. Includes adjustable regulator, pressure gauge, pressure switch, and foot pedal. NOTE: Programmable Air is required to operate Haas Air Vises on VF, DT, and DM series mills. Not compatible with the VF-2YT series Pallet Pool (PALLET POOL-VMC-SM). Not compatible with the Automatic Air Clamping Provision (AIR CLAMP PRO-AUTO) on Haas tilting rotary tables. |
| HSM | High-Speed Machining |
| RJH-TOUCH-XL | Remote Jog Handle XL |
| PS10CT | 10 Pull Studs, CT40 |