1. Product Overview:

| No. | Technical Specifications | Requirements | Note |
| --- | --- | --- | --- |
| 1 | Printer Performance |  |  |
|  | Print Mode | Thermal Transfer / Direct Thermal |  |
|  | Print Resolution | 300dpi |  |
|  | Label Size | Custom |  |
| 2 | Labeling Performance |  |  |
|  | Labeling Method | Tamp Applicator |  |
|  | Labeling Direction | Front Face / Back Face |  |
|  | Labeling Mode | Print & Apply |  |
|  | Repeat Labeling Accuracy | ±3mm | Varies with product |
| 3 | Others |  |  |
|  | Conveyor Height |  |  |
|  | Conveyor Speed |  |  |
|  | Labeling Height Adjustment | 900~1400mm | To be determined by design |

2. Printer Specifications

| Paper exit direction |  | right side |
| --- | --- | --- |
| Printing method |  | thermal transfer / direct thermal |
| Print resolution |  | 300 dpi |
| Maximum print speed |  | 254mm/s |
| Maximum print width |  | 168.0mm |
| Printable area |  | 2 mm from label edge |
| Label material |  | paper, hang tag cardstock, plastic materials (PET, PE, PP, PI, PVC, PU, Acrylate) |
| Paper roll | Paper roll type | Paper roll type |
|  | Maximum outer diameter | 300 mm |
|  | Core diameter | 76 mm |
| Label | Minimum height | 6 mm |
|  | Width | 20-170mm |
|  | Thickness | 0.03 - 0.6mm |
| Label backing | Width | 24-170mm |
|  | Thickness | 0.03 - 0.16mm |
| Ribbon | Ribbon orientation | outside wound / inside wound |
|  | Maximum outer diameter | 90 mm |
|  | Inner diameter | 25.4mm |
|  | Maximum length | 600 mm |
|  | Ribbon width | 25-170mm |
| Label sensor | Through-type sensor | detects label front edge, punched mark, label rear edge, or black mark on transparent labels. |
|  | Reflective sensor | detects label front edge, label rear edge, or black mark on transparent labels. |
|  | Sensor distance | 5-60mm |
|  | Maximum detectable height of the sensor | 2mm |
| Data interface | RS232C serial port | 1,200 to 230,400 Baud / 8 Bit |
|  | Ethernet | 10/100 Mbit/s |
| Pattern | Graphic elements | lines, arrows, rectangles, circles, ellipses, filled shapes |
|  | Pattern format | PCX, IMG, BMP, TIF, MAC, GIF, PNG |
| Barcode | 1D barcode | Code 39, Code 93 Code 39 Full ASCII Code 128 A, B, C |
|  | 2D barcode and stacked 2D barcode | DataMatrix DataMatrix Rectangle Extension QR-Code Micro QR-Code GS1 QR-Code GS1 DataMatrix PDF 417 Micro PDF 417 UPS MaxiCode GS1 DataBar Aztec Codablock FDotcode RSS 14 truncated,limited,stacked, stacked omni-directional |
| Operating environment | Operating voltage | 100 - 240 VAC, 50/60 Hz |
|  | Power consumption | standby < 100 W / normal operation 400 W |
|  | Operating conditions | -5 to 40 °C / 10–85 % RH, non-condensing |
|  | Storage conditions | 0 to 60 °C / 20–85 % RH, non-condensing |
|  | During transport | –25 to 60 °C / 20–85 % RH, non-condensing |

3.Labels and ribbons for automatic printing

| No. | Type | Parameter | Note |
| --- | --- | --- | --- |
| 1 | Label | Label material | coated paper (provided by buyer) |
|  |  | Label thickness | ≥ 50 µm |
|  |  | Label size | L1 = 250 mm, W2 = 150 mm |
|  |  | Label spacing L0 | ≥ 2 mm |
|  |  | Label margin W1 | ≥ 2 mm |
|  |  | Backing paper width W | 100 mm ≤ W ≤ 170 mm |
|  |  | Label roll core inner diameter d | 76mm |
|  |  | Label roll core outer diameter D | 300mm |
| 2 | Sample |  |  |
| 3 | Label usage requirements | 1. Labels for automatic printing differ from manual ones. Backing paper must have no tear lines, and only labels tested and certified by us for material, thickness, translucency, release force, adhesion, and die-cut accuracy are qualified. |  |
|  |  | 2. The buyer must provide the actual labels for factory testing within 2 weeks after signing the agreement, with no less than 2,000 sheets per label type for each printing and labeling machine. |  |
|  |  | 3. Labels not certified by us may be incompatible with the printing and labeling machine and are not covered by the product warranty. |  |

4.Main configuration

| No. | Equipment | Brand / Origin |
| --- | --- | --- |
| 1 | Print engine | Zebra, USA |
| 2 | Cylinder | AIRTAC ,Taiwan |
| 3 | PLC | Panasonic,Japan |
| 4 | Solenoid valve | Festo,Germany |
| 5 | Vacuum generator | Festo,Germany |
| 6 | Air filter | Chinese brand |
| 7 | Switching power supply | Omron,Japan |
| 8 | Reflective sensor | SICK,Germany |
| 9 | Switching power supply | Mean Well,Taiwan |
| 10 | Touch screen | XianKong / Kunlun Tongtai,China |