   

[image: image7.jpg]



[image: image7.jpg]

HIGH PRESSURE FEEDER  REPORT

                                                                                                                                           (REV. 9  02/18/2010  B.L.Guy)
RECEIVING INSPECTION REPORT
JOB#: 50384773
DATE: 11/20/2012
INSPECTOR:  B.L.Guy    
SIZE: 260
CUSTOMER & LOCATION: Lincoln P&P  Lincoln, ME
S.H.#: PC # 3018
KAMYR#:      
MYRENS#:      
GENERAL RECEIVING INFORMATION:

FEEDER SKID: YES
COMMENTS: N/A                Feeder Settings @ Shipment--

STEAM PIPES: NO
COMMENTS: N/A                Cold Travel-     
EQUALIZATION PIPES: NO
COMMENTS: N/A                Rotor Setting-              

HANDWHEEL: YES
COMMENTS: N/A
SCALE: YES
COMMENTS: N/A

SCALE READING AS RECEIVED: N/A
FEET: YES
COMMENTS: N/A
COUPLING: YES
COMMENTS: N/A

LOCATION FROM END OF SHAFT: Set flush.
FLINGERS: YES 
COMMENTS: N/A
LANTERN RINGS:NO             COMMENTS:N/A
LIFTING PLATES: YES
COMMENTS: Bolt on top
GENERAL REMARKS:      

HARDWARE INSPECTION LIST
T.S. HEAD: REPLACE
NUM & SIZE: 24 ea 1 1/4"-7 X 3 1/8" lg
D.S. HEAD: REPLACE
NUM & SIZE: 12 ea 1"-8 X 3" lg
LOCKPIN ASSY.: YES
NUM & SIZE: N/A
HANDWHEEL BOLT: YES
NUM & SIZE: N/A
PACKBOX: REPLACE
NUM & SIZE: 8 ea 3/4"-10 X 4 1/4" lg
PACKING GLAND: YES
NUM & SIZE: N/A
FEEDER FEET: OK
NUM & SIZE: N/A
HIGH PRESSURE FEEDER REBUILD REPORT
ROTOR HOUSING

[image: image1.wmf]SECTION A-A

1a

1b

3a

3b

1

4

3

2

2a/4a

2b/4b

12

13

6

9

11

10

A

B

13

12

1

2

3

4

5

7


A) CONDITION OF LINER BORE: Poor
CRACKS FORMCHECKBOX 

EROSION FORMCHECKBOX 

POROSITY FORMCHECKBOX 




BORE SIZE AS RECEIVED:30 1/2"
ACTUAL FINISHED DIAMETER:     


B) LINER THICKNESS AS RECEIVED:1/2"
FINISHED THICKNESS:     

LINER  MATERIAL: Is magnetic
LINER HARDNESS(BHN): T.S.     
MID.     
D.S.     
1) SEAL WELDS: 1a FORMCHECKBOX 
   1b FORMCHECKBOX 
   2a FORMCHECKBOX 
   2b FORMCHECKBOX 
   3a FORMCHECKBOX 
   3b FORMCHECKBOX 
   4a FORMCHECKBOX 
   4b FORMCHECKBOX 


COMMENTS: Also has liner cracks in the areas denoted



2) POCKET SKINS: 1a FORMCHECKBOX 
   1b FORMCHECKBOX 
   2a FORMCHECKBOX 
   2b FORMCHECKBOX 
   3a FORMCHECKBOX 
   3b FORMCHECKBOX 
   4a FORMCHECKBOX 
   4b FORMCHECKBOX 


COMMENTS: S/S overlaly, S/S ground away to fit grids.



3) SIDE FLANGES: 1a FORMCHECKBOX 
   1b FORMCHECKBOX 
   2a FORMCHECKBOX 
   2b FORMCHECKBOX 
   3a FORMCHECKBOX 
   3b FORMCHECKBOX 
   4a FORMCHECKBOX 
   4b FORMCHECKBOX 


COMMENTS: 95 % Stl , see pics



4) FLANGE SKINS: 1a FORMCHECKBOX 
   1b FORMCHECKBOX 
   2a FORMCHECKBOX 
   2b FORMCHECKBOX 
   3a FORMCHECKBOX 
   3b FORMCHECKBOX 
   4a FORMCHECKBOX 
   4b FORMCHECKBOX 


COMMENTS: 80 % Stl, see pics



5) POCKET WEDGES: 2a FORMCHECKBOX 
   2b FORMCHECKBOX 
   4a FORMCHECKBOX 
   4b FORMCHECKBOX 
   


COMMENTS: OK, wedge plates, see pics



6) T.S. GASKET FACE: OK
CONDITION:N/A
7) D.S. GASKET FACE: OK
IS OVERLAY REQUIRED? NO
8) STEAM DISTRIBUTION TYPE: See pics
9) DRAIN PIPES: NO
TYPE:      
COMMENTS:See pics
10) STEAM PIPES: NO
TYPE:      
COMMENTS:See pics
ROTOR HOUSING CONTINUED:

11) STEAM JACKETS: See pics
LOCATION:N/A
12) SCREEN GRIDS: GRID (3a) OK
THICKNESS:1 1/6"
COMMENTS:N/A

GRID (3b) REPAIR
THICKNESS:1 1/6"
COMMENTS:Cracked thru, see pics
13) GRID BLOCKS: OK
COMMENTS:N/A
14) HOUSING EROSION - ESTIMATED UNSUPPORTED AREA : 30%
15) CONDITION OF TAPPED HOLES IN HOUSING:


TOP FLANGE(1): OK
NUMBER & SIZE:N/A

SIDE FLANGE(2): OK
NUMBER & SIZE:N/A

BOTTOM FLANGE(3): OK
NUMBER & SIZE:N/A

SIDE FLANGE(4): OK 
NUMBER & SIZE:N/A

T.S. FLANGE: OK
NUMBER & SIZE:N/A 
           FEET BOLT HOLES (420&260) : OK                   NUMBER & SIZE: N/A    
16) T.S. PILOT DIAMETER: 33.862
CONDITION:OK

FINAL PILOT DIAMETER:      
17) D.S. PILOT DIAMETER: 21.026
CONDITION:OK

FINAL PILOT DIAMETER:      
18) 2” PIPE PLUGS: NO                         
CONDITION:
N/A
19) RELIEF GROOVES: 


NUMBER PER POCKET: 4
TYPE: Tapered 


WIDTH: 3/8"
DEPTH: 1/4"
LENGTH: 3"
(AS RECEIVED)


WIDTH:      
DEPTH:      
LENGTH:      
(AFTER REBUILD)






2"
3
3 1/2"




10 1/2"

NOTE: SEE ATTATCHED N.D.T. REPORT(S) FOR HOUSING

GENERAL REMARKS:      
ADJUSTING SIDE END HEAD

  [image: image2.wmf]20

22

23 & 24

C

D

E

F

G

21



AS RECEIVED
CONDITION
AS FINISHED

C) BEARING BORE: 
9.846/.847
Damaged , brg failure
     
D) SEAL BORE: 
7.901
Has a previous repair,damaged from brg failure,WILL REQUIRE REPAIR AGAIN,Note:is a bolt on repair, see pics
     
E) PACK-BOX BORE: 
7.8715/.8745
Erosion to face of box, see pics
     
F) WEAR RING BORE: 
6.364/.368
Out of round
     
G) PILOT DIAMETER: 
33.861
OK
     
20) GASKET FACE: OK
IS OVERLAY REQUIRED? NO 

21) EQUALIZATION PIPE: YES
COMMENTS: 1 3/8" pipe
22) BACK SIDE OF HEAD: REPAIR
CONDITION: Eroded 

23) COOLING WATER: NO
COMMENTS: N/A



WATER HOLES: (SIZE & CONDITION) N/A
24) SEAL WATER: NO
COMMENTS: N/A
25) TAPPED HOLES: REPAIR
NUMBER & SIZE:1 ea 3/4"-10 x 
26) 2” PIPE PLUGS: NO
CONDITION: N/A


COMMENTS: N/A
GENERAL REMARKS:See pics for a crack, and threaded hole at top quadrant of main flange.// Erosion to face of pk box and face and O/D of main flange,see pics.
DRIVE SIDE END HEAD

   [image: image3.wmf]J

K

M

H

N

27

28

29

30 & 31



AS RECEIVED
CONDITION
AS FINISHED

H) BEARING BORE: 
9.848
Eroded
     
J) SEAL BORE: 
7.8805
Very pitted, see pics
     
K) PACK-BOX BORE: 
7.894/.895
OK
     
M) WEAR RING BORE: 
6.383
OK
     
N) PILOT DIAMETER: 
21.024
OK
     
27) GASKET FACE: OK
CONDITION: N/A
28) EQUALIZATION PIPE: NO
TYPE: N/A
COMMENTS: Modified to D/S of main hsg.
29) BACK SIDE OF HEAD: REPAIR
CONDITION: Eroded
COMMENTS: N/A
30) COOLING WATER: NO
COMMENTS: N/A

WATER HOLES: (SIZE & CONDITION) N/A

HAS PACKING GLAND BEEN ROTATED (650L)? NO
31) SEAL WATER: NO
COMMENTS: N/A
32) TAPPED HOLES: REPAIR
NUMBER & SIZE:1 ea at pk. box, cracked, see pics
GENERAL REMARKS:     
PACK-RYT INFORMATION Form

                                                                                                                                           REV. 1  01/16/2008 B.L.Guy

TENDING SIDE END HEAD:

PACK BOX BORE:                                                  See pg # 4
PACKBOX BORE LENGTH:                                   4 1/16" With wear ring
WATER HOLE DIAMETER:                                    N/A
WATER HOLE LOCATION:                                     N/A
(FROM TOP OF WEAR RING TO C/L OF HOLE)

WEAR SLEEVE DIAMETER:                                  See print
DRIVE SIDE END HEAD:

PACK BOX BORE:                                                  See pg # 5
PACKBOX BORE LENGTH:                                   4 1/16" With wear ring
WATER HOLE DIAMETER:                                    N/A
WATER HOLE LOCATION:                                     N/A
(FROM TOP OF WEAR RING TO C/L OF HOLE)

WEAR SLEEVE DIAMETER:                                  See print
ROTOR

[image: image4.wmf]Q

R

P

TENDING SIDE

DRIVE SIDE

T

U

S



AS RECEIVED
CONDITION
AS FINISHED

P) DIAMETER OF LARGE END: 
30 3/8"
N/A
     
Q) T.S. PILOT DIAMETER: 
8.887/.888
N/A
     
R) D.S. PILOT DIAMETER: 
8.718
N/A
     
S) OVERALL ROTOR LENGTH: 
39"
N/A
     
T) ROTOR FACE TO PILOT FACE(T.S.): 1 1/2"
N/A
     
U) ROTOR FACE TO PILOT FACE(D.S.):1 1/2"
N/A
     
33) TAPPED HOLES: OK
NUMBER & SIZE:N/A
34) JOURNAL BOLTS: REPLACE
NUMBER & SIZE: 24 ea 1 1/4"-7 X 3" lg
35) VISIBLE CRACKS: NO
LOCATION & DESCRIPTION: N/A
36) NUMBER OF OVERLAYS: N/A
RADIOGRAPHIC INSPECTION:      

HARDNESS(BHN) AS RECEIVED: 
T.S.     
MID.     
D.S.     

HARDNESS(BHN) AFTER REBUILD: T.S.     
MID.     
D.S.     
NOTE: SEE ATTATCHED N.D.T. REPORT(S) FOR ROTOR.

GENERAL REMARKS:     
JOURNALS

ADJUSTING JOURNAL

             [image: image5.wmf]Y

W

V

X

37

Z


FLANGE TYPE: Sml
ADJUSTING JOURNAL:

AS RECEIVED
CONDITION
AS FINISHED

V) THRUST BEARING FIT:
3.0655/.1515
Severely damaged, brg. failure, see pics
     

OLD STYLE FIT:
N/A
N/A
N/A

THREADS: Chase
COMMENTS: Nut missing
W) MAIN BEARING FIT:
5.7015
Damaged from brg. failure
     

THREADS: Chase
COMMENTS: Nut missing
X) WEAR SLEEVE DIAMETER:
REPLACE
Worn
     

WEAR SLEEVE FIT:
5.970/.972
OK
     
Y) PILOT FIT:
8.885
OK
     
Z) BRG. SHOULDER TO FLG.:
15" 
OK
     
37) FLANGE: OK
THICKNESS: 1 7/16"
COMMENTS: N/A
38) SEAL FIT (650L): 
N/A
N/A
N/A
39) RADIUS COVER: Full cover
COMMENTS: Removed for inspection

RADIUS INSPECTION: YES
INSPECTOR: 107026
COMMENTS: Inconclusive


NOTE: SEE ATTACHED N.D.T. REPORTS FOR JOURNALS.

GENERAL COMMENTS:      
DRIVE JOURNAL

[image: image6.wmf]AE

41

AA

AB

AC

AD

42

AF


FLANGE TYPE: Sml
DRIVE JOURNAL:


AS RECEIVED
CONDITION
AS FINISHED

AA) DRIVE FIT:
4.724
Minor dings
     
AB) SEAL FIT:
4.877
Grooved .030/.040, bad dings
     

SEAL FIT ADAPTER(260L): N/A
COMMENTS: N/A
AC) MAIN BEARING FIT:
5.309
Damaged, brg. failure,see pics
     

THREADS: Chase
COMMENTS: No nut
AD) WEAR SLEEVE DIAMETER:
REPLACE
Worn
     

WEAR SLEEVE FIT:
5.975/.976
OK
     
AE) PILOT FIT:
8.717
OK
     
AF) BRG. SHOULDER TO FLG.:
15"
OK
     
40) FLANGE: Minor erosion
THICKNESS: 1 3/8"
COMMENTS: Face and O/D
41) RADIUS COVER: Full
COMMENTS: Removed for inspection

RADIUS INSPECTION: YES
INSPECTOR: 107026
COMMENTS: Inconclusive
42) KEYWAY:
1 1/4"X 1/4"X 8 13/16"
OK
     
NOTE: SEE ATTACHED N.D.T. REPORTS FOR JOURNALS.

GENERAL COMMENTS:      
ADJUSTING ARRANGEMENT AND

MISCELLANEOUS COMPONENTS

43) ADJUSTING ASSEMBLY HOUSING: REPAIR
COMMENTS: Lockpin threads bad
44) LOCKPIN ASSEMBLY: YES, REPAIR
COMMENTS: Threads bad, slot pin damaged, see pics
45) ADAPTER (THRUST BRG.) HOUSING: YES, REPAIR
COMMENTS: Slot damage 360 Deg's // damage to spring retainer area, see pics for all

SNAP RING: N/A
46) SPINDLE: Bolt on
IF BOLT ON, GIVE NUMBER AND SIZE OF BOLTS: 16 ea 3/8"-16 Not rec'd

LARGE DIAMETER THREADS: >>>>>>>>>>>>
COMMENTS: Flange thickness is 3/4"

SMALL DIAMETER THREADS: OK
COMMENTS: Die Gage Fit - Won't go

SPINDLE HANDWHEEL FIT: 1.962
COMMENTS: OK

SPINDLE HANDWHEEL KEYWAY:5/8"X 5/32"X 2"
COMMENTS: OK

SPINDLE HANDWHEEL KEY: Not rec'd
COMMENTS: REPLACE
47) SPINDLE BUSHING (BRASS NUT): OK
COMMENTS: Tap gage Fit - Loose

FIT WITH SPINDLE: AXIALLY: .010
RADIALLY: 014
48) HANDWHEEL: YES
COMMENTS:OK

HANDWHEEL BORE: 1.963
COMMENTS: OK

HANDWHEEL KEYWAY: 5/8"X 9/16"X Thru
COMMENTS: OK
49) PACKING GLANDS: YES
MAT’L: Stl
COMMENTS: 3 pc, OK

GLAND HARDWARE: N/A
DESCRIPTION: N/A
50) LANTERN RINGS: NONE
MATERIAL: N/A
COMMENTS: N/A
51) THRUST BEARING NUT: REPLACE
COMMENTS: Not rec'd
52) D.S. BEARING COVER: YES
COMMENTS: OK

SEAL BORE AS RECEIVED:5.9115
SEAL BORE AS FINISHED:     
53) REMOVAL SLEEVE: REPLACE
COMMENTS: N/A
54) BEARING SHOULDER RINGS: N/A        COMMENTS: N/A
55) DRIVE APPARATUS: Coupling
CONDITION: Not browns style 


BORE:
4.725/.727
COMMENTS: OK

KEYWAY: 
1 1/4"X 3/4"X Thru

COMMENTS: OK

KEY: 
1 1/4"X 1"X 8 3/16"

COMMENTS: OK

TAPPED HOLES: REPAIR
NUMBER & SIZE:1 ea bad heli-coil // 2 ea badthreads 1/2"-13
GENERAL REMARKS:      
Liquid Penetrant Inspection Form

                                                                                                                                           REV. 1  10/24/06 B.L.Guy

JOB#: 50384773          DATE: 11/20/2012      INSPECTOR:  B.L.Guy    
SIZE: 260          CUSTOMER & LOCATION: Lincoln P&T  Lincoln,ME          S.H.#: PC # 3018

 
SPECIFICATIONS:  Section # 5 ASME             COMPONENT: Rotor & Coupling                             

SOLVENT VISIBLE: XXX FORMTEXT 

     

             WATER WASHABLE:    
RESULTS: 

INSPECTOR CLOCK NUMBER: 907 

 ACCEPTED:XXX     REJECTED:                REMARKS:Rotor - No defects 

ACCEPTED:          REJECTED:XXX           REMARKS:CouplingCracks at set screw hole 

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:      

ACCEPTED:          REJECTED:                REMARKS:     
ACCEPTED:          REJECTED:                REMARKS:     
ACCEPTED:          REJECTED:                REMARKS:     
ACCEPTED:          REJECTED:                REMARKS:     
ACCEPTED:          REJECTED:                REMARKS:     
GENERAL COMMENTS:      
Component Scrap Request

                                                                                                                                           REV. 1  11/23/09 B.L.Guy

JOB#: 50384773          DATE: 11/20/2012      INSPECTOR: B.L.Guy
SIZE: 260          CUSTOMER & LOCATION: Lincoln P&T  Lincoln, ME          S.H.#: PC # 3018

 
INSPECTOR CLOCK NUMBER: 907 

Items scrapped : 

:Scale                REMARKS:Per R. Campbell 

:T/S Flinger                REMARKS:Per R. Campbell 

:D/S Flinger                REMARKS:Per R. Campbell 

:Lock pin                REMARKS:Per R. Campbell 

:T/S Journal                REMARKS:Per R. Campbell 

:D/S Journal                REMARKS:Per R. Campbell 

:                     REMARKS:      

:                     REMARKS:     
:                     REMARKS:     
:                     REMARKS:     
:                     REMARKS:     
:                     REMARKS:     
GENERAL COMMENTS:      
TENDING SIDE





DRIVE


 SIDE





_____ SPACES AT__________











       =___________








PAGE  
Page 12 of 9

