Part ID: 230400 - MAC Bucket with Rubber Auger

- Capacity

1/2 yard (13.5 cubic ft.); up to 3/4 yard (20 cubic ft.) with optional hopper extension

- Concrete Mix Capacity (80-lb bags)

Approx. 18 (1/2 yd)
Approx. 25 (3/4 yd)

- Dispense Rate

28 seconds @ 24 GPM

- Hydraulic Requirements

Up to 24 GPM
Up to 3,500 PSI

- Mounting Options

Skid-Steer Quick Attach

- Height, Grate Lowered (A) (inches)

33.5"

- Height, Grate Open (B) (inches)

47"

- Overall Width (C) (inches)

26"

- Overall Length (D) (inches)

77.4"

- Overall Hopper Length (E) (inches)

71.3"

- Weight

615 lb.

- Auger Diameter without Rubber (inches)

4"

- Auger Diameter with Rubber (inches)

6.5"

- Center Column Diameter (inches)

1.32" O.D.
.18" wall

- Auger Length (inches)

60"

- Flighting Spacing (inches)

4"